| 1/ Storage of Flammable substances - Fire - Contact with flammable substances | Event Staff | 8 | 4 | 32 | - Pyrotechnic cartridges to be stored in accordance with HSE regulations. - Storage facilities off-site. - MSDS Recommendations adhered to. - Appropriate PPE’s to be worn during use and handling. - Only trained personnel to use and operate the equipment. - Have appropriate fire extinguishers readily to hand | 2 |
| --- | --- | --- | --- | --- | --- | --- |
| 2/. Flammable Hazardous substances and use of pyrotechnical devices. - Toxic vapours - Fire and explosion | Event Staff | 6 | 4 | 24 | - Only trained personnel to use the equipment and pyrotechnic effects. - Cartridges to be stored in accordance with HSE regulations. - MSDS Recommendations adhered to. - Appropriate PPE’s to be worn during use and handling. - Extensive technical rehearsals. - Communication with those in vicinity of the operations. - No known toxic effects. - Advice medical personnel of possible problems for asthmatics. - Have appropriate fire extinguishers readily to hand | 2 |
| 3/. Fire - Injury to persons - Destruction of property | Event Staff | 6 | 2 | 12 | - Only trained personnel to use the equipment and pyrotechnic effects. - Appropriate PPE’s and clothing to be worn. - Surrounding materials to be fireproofed. - Equipment self-extinguishing. - Extensive technical rehearsals. - Minimise pyrotechnics on site. - Store equipment and Pyrotechnics in designated packaging; in secure area away from Escape routes and hazards. - Communicate requirements to the site safety officer and others in vicinity - Have appropriate fire extinguishers readily to hand | 2 |

| 4/. Tampering with the pyrotechnics and other unauthorised actions. - Unauthorised explosions - Risk of fire - Injury to performers, backstage personnel and operatives. | Event Staff / Visitor | 8 | 4 | 32 | - Minimise exposure time when pyrotechnics available. - Audit use. - Minimise access to pyrotechnics when in position - Keep guarded - Secure and check all circuitry - Limit safety key access to named holders. - Use minimum of 2# personnel in area at all times. - Double check placement of all pyrotechnic effects. - Have required extinguishers and trained personnel available - Carry out checklist before firing. | 2 |
| --- | --- | --- | --- | --- | --- | --- |
| 5/. Misplacement of pyrotechnics from the designated positions. - Injury to performers, backstage personnel and operatives. - fire | Event Staff | 6 | 4 | 24 | - Make all devices site specific wherever possible - Ensure devices placed and fused properly and check the integrity of the circuits. - Only trained personnel to use and place the equipment and pyrotechnic effects - Have separate check on all placed devices by second trained person as audit. - Have required extinguishers and trained personnel available - Carry out checklist before firing. | 2 |
| 6/. Misfires. - Injury to performers and operatives. - Fire | Event Staff / Public | 6 | 4 | 24 | - Misfiring pyrotechnics to be handled as per the MSDS sheet. - Circuitry to be checked at completion of show and before commencement of next as per the Check List. - Only trained personnel to use and place the equipment and pyrotechnic effects - Carry out checklist before firing. | 2 |

| 7/. Miscuing, premature or late firing. - Injury to performers, backstage personnel and operatives. - Fire | Event Staff | 6 | 4 | 24 | - Only trained personnel to use and place the equipment and pyrotechnic effects - Have required extinguishers and trained personnel available - Circuitry to be checked at completion of show and before commencement of next as per the Check List. - Extensive technical rehearsals. - Ensure minimum distance requirements met as per any manufacturers recommendations or risk assessment. - Ensure that the key holding firing personnel are familiar with the act and sequencing. - Production to give clear voice signal with agreed wording. - Equip the firing station with a means of monitoring the area. - Remove or protect all flammable materials. - Ensure clear means of escape, clearly signed and lighted. - Carry out checklist before firing. | 2 |
| --- | --- | --- | --- | --- | --- | --- |
| 8/. Break-in to the circuitry by unauthorised personnel or mechanical damage. - Misfiring - Injury to performers, backstage personnel and operatives - Fire | Event Staff | 8 | 4 | 32 | - Clearly mark and separate pyrotechnics circuits. - Use only qualified electricians and advise of wiring. - Check circuit integrity utilising built in test facilities. - Ensure firing box tamper proof. - Carry out checklist before firing. | 2 |
| 9/. Spilled pyrotechnic materials | Event Staff | 2 | 2 | 4 | - Handle as MSDS sheet and evacuate area. | 2 |

Any other points to note, risks and control measures

Safety is a priority for DJSPACEBAR.

DJSPACEBAR will trigger a pyrotechnic confetti cannon after the first dance. Prior to the event, appropriate spacing will be arranged to ensure that no obstacles or personnel will be in it’s way.  If our pre-assessment check deems unsafe use of any such pyrotechnix, then DJSPACEBAR will review all areas and consider alternative measures.

During the guests celebration dance, DJSPACEBAR will trigger a further 2 hand confetti cannons over guests. These are not pyrotechnic devices.

DJSPACEBAR will be responsible for the cleanup operation of these items.

| #### Severity | M Death | S Death | MajorI | 3Day | MinorI |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Certain 10 | 10 | 8 | 6 | 4 | 2 | Primary Risk average | Residual Risk average |
|  | 100 | 80 | 60 | 40 | 20 | 23.11 | 2 |
| V Likely 8 | 80 | 64 | 48 | 32 | 16 |  |  |
| Probable 6 | 60 | 48 | 36 | 24 | 12 |  |  |
| Possible 4 | 40 | 32 | 24 | 16 | 8 |  |  |
| Unlikely 2 | 20 | 16 | 12 | 8 | 4 |  |  |
| V Unlikely 1 | 10 | 8 | 6 | 4 | 2 |  |  |

100-48		Unacceptable								M Death		Multiple Deaths

40-32 		Risk reduction necessary						S Death		Single death

24-16 		Control measures essential						MajorI			Major Injury as defined by RIDDOR

12-2		Monitor								3Day			3 Days off sick or injured

MinorI			All other injuries

Es-Staff, Ep-performers, V-Visitors, C-Contractors, P-Public

This document has been supplied as a template by Le Maitre. Le Maitre cannot be held liable for any legal action resulting from the use of this document, Le Maitre have given no input to completed risk assessments.